TECHNICAL SPECIFICATION

TravelBook Hotels

NFC Glass Room Key System

────────────────────────────────────

Document Type:  Technical Specification

Version:  1.0

Date:  March 2026

Prepared by:  TravelBook Product & Engineering

Classification:  Internal — Confidential

# 1. Executive Summary

This document defines the technical specification for the TravelBook Hotels NFC Glass Room Key System — a premium, multi-function credential combining NFC-based room access, an Apple Wallet digital pass, and a TravelBook loyalty card into a single borosilicate glass card, cryptographically linked to each guest's TravelBook account.

The system replaces traditional plastic keycards and fragmented app-based access systems with a unified, secure, and brand-elevating guest credential. The glass card is rewritable, allowing the same physical card to be reprogrammed for each new guest while retaining its premium material quality.

System Summary Card Material: Borosilicate glass, CR80 format (85.6mm × 54mm × 0.8mm) NFC Standard: ISO/IEC 14443 Type A / MIFARE DESFire EV3 Encryption: AES-128, 3DES Digital Pass: Apple PassKit (iOS) + Google Wallet API (Android) Loyalty: Four-tier programme: Silver, Gold, Platinum, Diamond Account Link: JWT + OAuth 2.0, HSM-backed key storage

# 2. System Architecture

The TravelBook Glass Key system comprises three integrated layers: the physical NFC card, the backend key management and loyalty service, and the digital pass layer (Apple Wallet / Google Wallet). These layers communicate via encrypted APIs and are unified under the TravelBook Account platform.

## 2.1 Architecture Overview

The system follows a layered architecture with each component independently scalable:

Physical Card Layer: borosilicate glass card with embedded NFC antenna and MIFARE DESFire EV3 chip

Key Management Service (KMS): HSM-backed service responsible for provisioning, rotating, and revoking NFC sector keys

TravelBook Account Service: guest identity, loyalty ledger, booking data, and card-to-account binding

Digital Pass Service: generates and maintains Apple Wallet and Google Wallet passes via PassKit and Google Pay API

Property Management System (PMS) Integration: bridges the KMS with the hotel's door control system via OSDP v2

Mobile App / Web Interface: guest-facing booking, card management, loyalty redemption, and pass delivery

## 2.2 Data Flow

The end-to-end credential lifecycle follows these steps:

Step 1 – Booking: Guest completes reservation via TravelBook app or website. A booking token is issued and linked to the guest's account UUID.

Step 2 – Key Provisioning: At T-24 hours before check-in, the KMS generates a time-bound NFC sector key pair (AES-128) scoped to the assigned room. The key is stored in the HSM and a key handle is returned to the account service.

Step 3 – Digital Pass Delivery: The Digital Pass Service constructs an Apple Wallet pass (pkpass bundle) containing the room number, guest name, loyalty tier, check-in/out dates, and a signed barcode. The pass is delivered via APNs push notification.

Step 4 – Physical Card Provisioning: At check-in, the front desk agent uses the Card Writer Terminal to write the provisioned NFC key handle and guest profile data to the glass card's MIFARE DESFire application directory. The card is linked to the booking in the account service.

Step 5 – Room Access: The guest taps the glass card or their iPhone (Express Mode) against the door reader. The door controller queries the OSDP gateway, which validates the one-time challenge-response token against the KMS. On success, the door unlocks and the tap event is logged.

Step 6 – Loyalty Crediting: Each validated tap event triggers a loyalty event in the account service. Points are credited in real-time based on the guest's tier multiplier.

Step 7 – Check-Out: On check-out, the KMS revokes the room's key sector. The Apple Wallet pass is updated via APNs to show 'checked out' status. The physical card's NFC sectors are wiped and the card is collected for reuse.

# 3. NFC Card Specification

## 3.1 Physical Specification

| Parameter | Specification |
| --- | --- |
| Form Factor | CR80 — 85.6mm × 54mm × 0.8mm |
| Material | Borosilicate glass (ISO 10373 compliant) |
| Antenna | Embedded copper NFC antenna, single-loop |
| Chip | NXP MIFARE DESFire EV3 (MF3D H40) |
| Memory | 8KB EEPROM, multi-application (ADF) |
| Operating Frequency | 13.56 MHz (HF NFC) |
| Communication Standard | ISO/IEC 14443 Type A |
| Read Range | ≤ 4cm (compliant with ISO 10373-6) |
| Operating Temperature | -25°C to +70°C |
| Card Lifespan | Write endurance: 500,000 cycles; retention: 10 years |
| Personalisation | Laser-etched guest name, room number, tier badge |
| Rewritability | NFC sectors wiped and reprinted per guest cycle |

## 3.2 NFC Application Structure

The glass card uses the MIFARE DESFire Application Directory (ADF) model to support multiple logical applications on a single chip. This allows room access, loyalty data, and guest identity to coexist securely in isolated application containers.

| Application | AID | Contents | Encryption |
| --- | --- | --- | --- |
| Room Access App | 0xAA0001 | Room number, access zones, valid from/to timestamps, one-time token seed | AES-128 |
| Guest Identity App | 0xAA0002 | Booking ID, account UUID, guest name hash, tier code | AES-128 |
| Loyalty App | 0xAA0003 | Points balance, tier level, stay counter, last updated timestamp | 3DES |
| Emergency App | 0xAA0004 | Emergency contact reference, medical flags (optional, guest-set) | AES-128 |

## 3.3 Security Model

The card security architecture is based on the following principles:

Per-Card Diversified Keys: Each card instance holds a unique diversified key derived from a master key using CMAC-based key diversification (per NXP AN10922). No two cards share the same application key.

One-Time Authentication Tokens: Door challenge-response uses a rolling token derived from the session key and a monotonic counter. Replayed tokens are rejected.

Application Isolation: The MIFARE DESFire ADF model ensures that the room access application cannot be read or written to by the loyalty reader, and vice versa.

Time-Bounded Access: Key validity windows are encoded in the access application. Expired keys are rejected by the reader before contacting the KMS.

Revocation: The KMS can push a key revocation to all door readers via the OSDP gateway within 500ms of receiving a revocation request.

# 4. Apple Wallet Integration

The digital pass is implemented using Apple PassKit (iOS 6+) and delivered to the guest's Apple Wallet via APNs push notifications. The pass type used is boardingPass, which provides the most feature-rich layout for hotel credential use cases.

## 4.1 Pass Structure

| Pass Field | Value / Source | Notes |
| --- | --- | --- |
| passTypeIdentifier | pass.com.travelbook.roomkey | Registered with Apple Developer Portal |
| teamIdentifier | TravelBook Hotels Ltd team ID | Organisation's Apple Developer Team ID |
| serialNumber | booking_uuid + '-' + room_number | Unique per booking instance |
| description | "TravelBook Room Key" | Shown in Wallet list view |
| headerFields | Guest name, Tier badge | Visible in stacked Wallet view |
| primaryFields | Room number | Large display on pass face |
| secondaryFields | Check-out date, Floor | Below primary field |
| auxiliaryFields | Points balance, Nights status | Below secondary fields |
| backFields | Booking ID, Access zones, T&Cs | Reverse of pass (tap ⓘ) |
| barcode | PDF417, encodes booking UUID | Fallback for non-NFC readers |
| nfc | Encrypted NFC payload, requires auth | Express Mode capable |
| expirationDate | Check-out date + 24h | Pass auto-expires |

## 4.2 NFC Express Mode

The pass is configured with the nfc key in pass.json, enabling iPhone Express Mode. This allows the guest to tap their iPhone to unlock the room without Face ID / Touch ID, even when the battery is critically low. The encrypted NFC message is signed with an ECDH P-256 ephemeral key pair and validated by the door reader's PassKit-compatible NFC module.

## 4.3 Pass Updates

Pass updates are delivered via APNs using the webServiceURL and authenticationToken fields in pass.json. The following events trigger a pass update push:

Room assignment change (upgrade or downgrade)

Check-out completed (pass marked inactive)

Loyalty points credited (balance updated)

Early check-in activated

Late check-out granted

Emergency card suspension

# 5. Loyalty Programme Integration

The TravelBook loyalty programme is embedded directly in both the physical card and the Apple Wallet pass. The loyalty engine runs as a real-time microservice within the TravelBook Account Service.

## 5.1 Tier Structure

| Tier | Qualifying Nights/Year | Points Multiplier | Key Perks |
| --- | --- | --- | --- |
| Silver | 0–9 | 1.0× | Free Wi-Fi, early check-in requests, welcome drink |
| Gold | 10–29 | 1.5× | Guaranteed room upgrade, complimentary breakfast, 2× weekend bonus |
| Platinum | 30–59 | 2.0× | Suite upgrades, lounge access, late check-out guaranteed, priority housekeeping |
| Diamond | 60+ | 3.0× | Dedicated concierge, complimentary minibar, arrival gift, transfer service |

## 5.2 Points Earning Events

| Event | Points Awarded | Notes |
| --- | --- | --- |
| Room night stay | £1 spend = 10 base points × multiplier | Applied at check-out |
| F&B spend (hotel restaurant/bar) | £1 = 8 points × multiplier | Charged to room or direct |
| Spa & wellness spend | £1 = 8 points × multiplier | Linked via property POS |
| Card first use (new member) | +500 bonus points | One-time, on first NFC tap |
| App booking (no OTA) | +200 bonus points | Direct booking incentive |
| Referral (new TravelBook member) | +1,000 bonus points | Per successful referral |
| Loyalty card tap (non-stay) | +5 points | Dining or event visits |

## 5.3 Points Redemption

Points can be redeemed against:

Room rate discount: 1,000 points = £10 off room rate

F&B vouchers: 500 points = £5 credit

Room upgrades: 2,000 points per night upgrade

Partner rewards: flights, car hire, dining (via TravelBook partners)

Donation to charity partners: 100 points = £1 equivalent donated

# 6. Account Linking & Security

## 6.1 Card-to-Account Binding

Each physical glass card is linked to a specific guest booking via a binding record in the TravelBook Account Service. The binding uses the card's UID (7-byte, factory-locked) combined with the booking UUID and a provisioning timestamp to create a tamper-evident chain of custody.

## 6.2 Authentication Flow

The card authentication sequence at the door reader is as follows:

1. Reader broadcasts ISO 14443-3 REQA. Card responds with ATQA.

2. Anti-collision loop completes; reader obtains card UID.

3. Reader selects the Room Access Application (AID 0xAA0001) using MIFARE DESFire SelectApplication.

4. Mutual three-pass AES authentication (ISO/IEC 7816-4 compliant) establishes a session key.

5. Reader issues a challenge nonce. Card responds with a MAC derived from the session key and a monotonic counter.

6. Reader forwards the response to the KMS gateway (OSDP v2 encrypted channel).

7. KMS validates the MAC and the monotonic counter (replay attack prevention). On success, door unlock signal sent.

8. KMS logs the access event (timestamp, card UID, room ID, result) to the audit ledger.

## 6.3 Key Revocation

Card revocation is handled by the KMS. When a revocation is triggered (guest request, loss report, check-out, or security alert), the KMS marks the key handle as revoked and broadcasts the updated revocation list to all OSDP gateways in the property within 500ms via WebSocket. Door readers maintain a local cache of active keys, which is invalidated on revocation broadcast.

## 6.4 Data Privacy (GDPR)

The following data minimisation principles apply:

The NFC chip stores only a guest name hash, booking ID, and access parameters — no PII in plaintext.

Full guest profile data is held in the TravelBook Account Service (cloud), not on the card.

Card data is wiped on check-out; the guest's loyalty and account data is retained per the TravelBook Privacy Policy.

Apple Wallet passes are delivered directly to the guest's device via APNs; TravelBook does not retain a copy of the pkpass bundle after delivery.

Audit logs (door access events) are retained for 90 days per security policy, then anonymised.

# 7. Integration Requirements

## 7.1 Property Management System (PMS)

Integration with the hotel PMS is required for the following functions:

| PMS Event | TravelBook Action | Protocol |
| --- | --- | --- |
| New reservation confirmed | Trigger key pre-provisioning T-24h | REST webhook |
| Check-in completed | Activate card + deliver Wallet pass | REST API |
| Room change / upgrade | Re-provision card + update Wallet pass | REST API + APNs |
| Check-out processed | Revoke key + archive card + close loyalty stay | REST API |
| No-show / cancellation | Cancel provisioning + notify guest | REST API |
| Lost key report | Immediate key revocation + new card issue | REST API (priority) |

## 7.2 Door Control System

The door lock system must support OSDP v2 (Open Supervised Device Protocol) for encrypted communication with the KMS gateway. Wiegand-only systems require a protocol bridge module. The following door reader features are required:

ISO/IEC 14443 Type A NFC reader (13.56 MHz)

MIFARE DESFire EV3 application support

OSDP v2 or IP gateway connectivity

Apple PassKit NFC module (or third-party equivalent) for Wallet pass Express Mode

Audit log buffer (minimum 1,000 events) for offline operation

# 8. Card Manufacturing & Personalisation

Glass NFC cards are manufactured by specialist suppliers capable of embedding NFC antennae into borosilicate glass. The following specification applies to manufacturing orders:

| Specification | Requirement |
| --- | --- |
| Glass Type | Borosilicate glass (SCHOTT D263T or equivalent) |
| Chip Inlay | NXP MIFARE DESFire EV3 (MF3D H40), embedded via hot-press lamination |
| Card Thickness | 0.76–0.84mm (ISO 7810 CR80 compliant) |
| Surface Finish | Frosted matte both sides; polished edges |
| Personalisation Method | CO₂ laser engraving for guest name, room number, tier icon |
| Minimum Order Quantity | 500 units per property per year (property stock) |
| Blank Card Stock | Pre-keyed with transport key; personalised on-property via card writer |
| Certification | CE, RoHS, ISO 10373-6, ISO 14443 |

# 9. Implementation Roadmap

| Phase | Duration | Milestones |
| --- | --- | --- |
| Phase 1 — Design & Procurement | Weeks 1–4 | Finalise card design; commission glass card manufacturer; Apple PassKit certificate application |
| Phase 2 — Backend Development | Weeks 3–10 | KMS API development; loyalty engine integration; PMS webhook connectors; OSDP gateway setup |
| Phase 3 — Pilot Property | Weeks 10–16 | One property deployment; door reader upgrade; staff training; soft launch with Gold+ members |
| Phase 4 — Digital Pass Launch | Weeks 12–16 | Apple Wallet pass live; Google Wallet pass live; App update with card management UI |
| Phase 5 — Full Rollout | Weeks 16–26 | All properties; loyalty programme live; marketing campaign; partner integration |

# 10. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | March 2026 | TravelBook Product & Engineering | Initial specification release |

TravelBook Hotels · NFC Glass Room Key System · Technical Specification v1.0 · March 2026